## Raspberry Pi 5 Dual Wi-Fi Optimization Guide

Optimized for: 10 units, high-density video streaming (Internal) & low-req Internet (USB Dongle)

### 1. Hardware Assignment Overview

To ensure maximum stability for 20+ streaming devices, we prioritize the internal Pi chip for heavy local traffic:

- wlan0 (Internal Pi Wi-Fi): Dedicated to Controllab (High-bandwidth video).

- wlan1 (Edimax USB Dongle): Dedicated to Free-TAU (Low-demand Internet).

### 2. Implementation Steps

Run these commands on your master Pi to configure the interfaces.

#### Configure Internal Video Network (wlan0)

Assign the internal Wi-Fi to your video network. We set a low metric for high priority but disable its use as an internet gateway:

Bash

sudo nmcli connection modify "Controllab" connection.interface-name wlan0 ipv4.route-metric 100 ipv4.never-default yes

#### Configure Internet-Only Network (wlan1)

Assign the USB dongle to the internet network. It will act as the default gateway for the web but will have lower priority for local traffic:

Bash

sudo nmcli connection modify "Free-TAU" connection.interface-name wlan1 ipv4.route-metric 400 ipv4.never-default no

### 3. Verify Routing Strategy

Apply the changes:

Bash

sudo nmcli connection down "Free-TAU" && sudo nmcli connection up "Free-TAU"

sudo nmcli connection down "Controllab" && sudo nmcli connection up "Controllab"

Run ip route to confirm the logic:

- Default via wlan1: All internet traffic leaves through the Edimax dongle.

- Internal Subnet via wlan0: Video traffic stays on the high-performance internal chip (Metric 100).

### 4. Preparation for Mass Cloning (10 Units)

Before creating your SD card image, you must generalize the connection files located in /etc/NetworkManager/system-connections/:

- Remove MAC Addresses: Open each .nmconnection file and delete the mac-address= line. This prevents the image from failing when the hardware ID changes on the next Pi.

- Lock Interface Names: Ensure the interface-name= field is set to wlan0 or wlan1 respectively to maintain your performance strategy.